Project CARS — Technical Note

The CARS Live Console

A discovery-driven, live-sync NOC for the IT/OT fabric

The differentiator, in one line:  most ICS/SDN demonstrations show a hand-drawn network diagram beside a live system. The CARS console instead renders the controller’s own live model — every node, wire, port, and state is discovered, so the picture physically cannot disagree with the network.

# 1. Why this is a genuine differentiator

A hand-drawn topology is an assertion — it says what the author believes the network is. It cannot catch a mis-cabling, a silent device, a spoofed identity, or a link that just failed. The CARS console is the opposite: it is a projection of ground truth. Every element on screen is derived, live, from the SDN controller’s own view of the data plane — the same view it uses to make security decisions. If the console shows it, the controller believes it; if the controller is wrong, the console shows the network’s reality anyway (a link goes red, a host vanishes, a spoof counter climbs).

This makes the console three things at once: an operator’s situational-awareness tool, a live proof that CARS’s internal model matches the physical fabric, and a debugging instrument that has already surfaced real faults a static diagram would have hidden (Section 5).

# 2. How it works — one source of truth to the screen

The console has no topology of its own and stores no state. It is a thin renderer over the controller’s read-only feeds. The pipeline is five stages:

Figure 1 — the sync pipeline: the OT fabric feeds the controller’s live model; six JSON feeds drive six parts of the live view.

- The OT fabric (Open vSwitch) emits OpenFlow events, LLDP, and packet-ins to the controller.

- The CARS controller builds a live model from them — switch sessions, discovered hosts, LLDP links with ports, port up/down, source-guard drop counters, and a decision audit.

- It exposes that model as six read-only JSON feeds on its Event API (:8080).

- A threaded proxy on the OT host (Dell #1) serves the single-page console and relays the feeds — so the browser never talks to the control plane directly (no cross-origin exposure).

- The browser renders a force-directed SVG and re-polls all six feeds every 1.2 s, reconciling the picture to whatever the controller now believes.

# 3. How every process is kept in sync

Each feed owns one part of the picture. Nothing is hardcoded; add a host or pull a cable and the corresponding element changes on the next 1.2 s tick.

| Live feed | Syncs | What you see |
| --- | --- | --- |
| /cars/status | Switches, guard, blocks | Switch nodes go grey when a dpid drops; src-guard / arp-guard armed badges; the active-enforcement list; the green/red controller-live dot. |
| /cars/hosts | Nodes | PLC/HMI/supervisory/attacker nodes appear and vanish as the controller learns them — deduped by MAC and auto-placed at their real access switch. |
| /cars/links | Wirings | Switch-to-switch links drawn from LLDP, each end labelled with its real discovered port (e.g. p3 ↔ p1); host links show the access port. Hovering a link reveals the full binding. |
| /cars/ports | Link health | A link turns red/broken the instant its port drops, and the device node greys — physical faults appear within ~1 s. |
| /cars/guard | Anti-spoof telemetry | A running count of spoofed IP/ARP packets dropped at ingress, per protected identity — the guard’s work made visible as numbers. |
| /cars/audit | Decisions + attacks | A dated feed of every brain decision (FORBIDDEN/OPERATIONAL/CRITICAL); a blocked conduit pulses its attacker node red with a glowing block-line to the victim. |

# 4. The engineering that makes the sync live and trustworthy

- Discovery, not configuration. Hosts come from passive ARP/IPv4 snooping (keyed by switch + MAC, so identical-IP cells stay distinct); links come from LLDP with port numbers; liveness comes from OpenFlow port-status. The console reads all of this — it declares none of it.

- Node placement is inferred, not fixed. Each device is placed at its true access switch using an edge-port heuristic (an access port carries one MAC; an uplink carries many), then a small force simulation lays the graph out and lets the operator drag/pin nodes.

- Reconciliation each tick. On every poll the renderer diffs the discovered set against what is drawn — adding new nodes, removing departed ones, and recolouring links/nodes from the latest port and block state — so the view converges to ground truth without a reload.

- Safe by construction. The browser only ever reads; the proxy is threaded (concurrent polls never block) and tolerates client disconnects; and it isolates the browser from the control plane. When the controller is unreachable the console flips to an explicit ‘controller DOWN’ state rather than showing stale green.

# 5. Proof it mirrors reality — faults a static diagram would hide

Because the console shows discovered truth, it has repeatedly caught things that a drawn diagram never could — each became a documented finding:

- Cell 2 isolation: no link to ovs2 — the map correctly showed Cell 2 unlinked, exactly matching the deferred physical transit.

- ARP flux: discovery flagged the insider’s MAC presenting the supervisor’s IP — a real multi-homed-host misconfiguration, fixed once seen.

- Clone-IP collision: both PLCs share an IP; the console’s MAC+switch keying kept them as two distinct nodes where an IP-keyed view would have merged them.

- Boot-time ARP probes: a rebooting PLC briefly announced 0.0.0.0; the console exposed it overwriting a real address, prompting a hardening fix.

- Restart blindness: after a controller restart the console’s empty host list revealed that persistent flows were suppressing re-discovery — driving the clean-slate-on-reconnect fix.

# 6. What it demonstrates, live

The console turns the whole thesis into something an examiner can watch happen in real time:

- Pull an OT cable (or admin-down a port) → that link turns red and the device greys, within ~1 s.

- Send a spoofed packet → the source-guard drop counter ticks up — the anti-spoofing defence, visible.

- Launch an attack → the offending node pulses red, a glowing block-line snaps to the victim, and the decision feed logs FORBIDDEN — the reactive loop, on screen.

- Ping from the supervisor to the same PLC → logged OPERATIONAL and left flowing — the criticality-aware policy, demonstrated side by side.

In short: the console is not a picture of the system — it is the system, watching itself.